| V1.0 12/01/2002 Compiled by Bill Cotter N4ALG |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cable 19D901739G1 Sorted by P2 |  |  |  |  |  |
|  | P702 | J601 |  |  |  |  |  |
| OK? | P2 | P1 |  | Function | Color(s) | Signal Description |  |
|  | 1 | 15 |  | !SQ DIS!/SQ ARM | Gra - cntr | With J605 shunted between pins 2 & 3 this is the squelch arm input. With J605 shunted between pins 1 & 2, when this pin is grounded, all squelch action is overriden and received audio is passed to the speaker. |  |
|  | 2 | 6 |  | Vol/Sq Lo | Shield for 1,4,5 | Volume Squelch ground/shield connection |  |
|  | 3 | 17 |  | Alert tone/RX PA | Yel-Grn | Input to the audio amplifier for time out timer alert tone. This pin also has squelched volume squelch high audio on it, but also be aware that it is also DC Biased to 4.5 Volts so AC couple to this if you want to use it for something. |  |
|  | 4 | 7 |  | Vol/Sql Hi | Grn-cntr | Volume Squelch output (low impedance non-demphasized, non-squelched audio) |  |
|  | 5 | 8 |  | Vol Arm | Blu-cntr | Audio amplifier input from external volume control |  |
|  | 6 | 27 |  | *SQ DIS/ HL2 | Org | Option specific. |  |
|  | 7 | 14 |  | Control A- | Shield for 3 | Negative power supply output. Leave this unconnected. |  |
|  | 8 | 1 |  | FB1 | Wht-Brn | BCD channel selector, A. Pulled up to +5V through a 10k resistor. |  |
|  | 9 | 2 |  | FB2 | Wht-Red | BCD channel selector, B. Pulled up to +5V through a 10k resistor. |  |
|  | 10 | 12 |  | CAS | Vio | Channel Activity indicator. Follows Carrier Squelch action only regardless of radio programming. 0 Volts when squelched, 9 volts (via a 10k pullup) when un-squelched. |  |
|  | 11 | 19 |  | A+ | #16-Yel | Positive power supply connection. This provides all power to the radio except for the RF Power Amplifier |  |
|  | 12 | 10 |  | !CG DIS!/serial pgm | Wht-Grn | Ground this line to disable Channel Guard and mute received audio based on carrier squelch only. Also used as serial control line during programming. |  |
|  | 13 | 16 |  | Ign A+ | Red | Positive power supply connection. Essentially a logic input to the radio but needs A+ for radio to work. |  |
|  | 14 | 11 |  | PTT | Wht | Push to talk. Ground to Transmit |  |
|  | 15 | 5 |  | Mic-Lo | Shield for 16 | Microphone input ground/shield connection |  |
|  | 16 | 9 |  | Mic-Hi | SF22-Brn | Microphone input |  |
|  | 17 | 18 |  | SPKR-Hi | #16-Grn | One side of the speaker. |  |
|  | 18 | 20 |  | SPKR-Lo | #16-Brn | The other side of the speaker. |  |
|  | 19 | 21 |  | A- | #16-Blk | Negative power supply connection |  |
|  |  | Cable 19D901739G1 Sorted by P3 |  |  |  |  |  |
|  | J703 | P601 |  |  |  |  |  |
| OK? | P3 | P1 |  | Function | Color(s) | Signal Description |  |
|  | 1 | 13 |  | PA Key | Org-Blu | Grounding this pin enables the audio amplifier but does not force open the received audio gate. Used to enable the speaker audio amplifier to generate an alert tone when the CCT timer expires. |  |
|  | 2 | 33 |  | Spare, HL8 | Org-Grn | Option specific. |  |
|  | 3 | 31 |  | Spare, HL6 | Yel-Vio | Option specific. |  |
|  | 4 | 35 |  | Spare | Org-Brn |  |  |
|  | 5 | 28 |  | CG-lo, HL3 | shield for 5 | Option specific. |  |
|  | 6 | 26 |  | CG-hi/HL1 | Wht-cntr | Option specific. DATA IN for some radios. |  |
|  | 7 | 23 |  | Spare | Grn | N/C |  |
|  | 8 | 22 |  | Spare | Yel-Vio | N/C |  |
|  | 9 | 36 |  | !RESET! | Org | Used for radio programming. Grounding this pin will put the microprocessor into RESET. |  |
|  | 12 | 34 |  | !RX MUTE! | Yel-Blu | Ground to mute receiver audio... Side benefit is that it can be used as a Channel Guard decode indicate output. Open Collector output with a 10k pullup to +9 Volts. Grounded by Q721 when no Channel Guard tone is detected. |  |
|  | 13 | 24 |  | Spare | Yel-Brn | N/C |  |
|  | 14 | 32 |  | Adv Change, HL7 | Yel-Blk | Used by scan heads to advance channel. Also used as serial data clock line for radio programming. |  |
|  | 15 | 25 |  | Spare | Wht-Yel | N/C |  |
|  | 16 | 29 |  | !STORE! | Wht-Org | Used for radio programming. Grounding this line will cause the EEPROM to save data to non-volitale memory. |  |
|  | 17 | 3 |  | FB3 | Wht-Blu | BCD channel selector, C. Pulled up to +5V through a 10k resistor. |  |
|  | 18 | 4 |  | FB4 | Wht-Vio | BCD channel selector, D. Pulled up to +5V through a 10k resistor. |  |
|  | 19 | 30 |  | FB5/Mode A/*B | Wht-Gra | Used to select between two EEPROM banks. Also used as serial data line for radio programming. |  |
|  |  | 37 |  | Battery A+ | #8-Red | High current positive power supply connection. Provides power to RF Power Amplifier only. |  |
|  |  | 38 |  | Battery A- | #8-Blk | High current negative power supply connection. Provides power to RF Power Amplifier only. |  |
|  |  | Cable 19D901739G1 Sorted by P1 |  |  |  |  |  |
|  | J601 | P702 | P703 |  |  |  |  |
| OK? | P1 | P2 | P3 | Function | Color(s) | Signal Description |  |
|  | 1 | 8 |  | FB1 | Wht-Brn | BCD channel selector, A. Pulled up to +5V through a 10k resistor. |  |
|  | 2 | 9 |  | FB2 | Wht-Red | BCD channel selector, B. Pulled up to +5V through a 10k resistor. |  |
|  | 3 |  | 17 | FB3 | Wht-Blu | BCD channel selector, C. Pulled up to +5V through a 10k resistor. |  |
|  | 4 |  | 18 | FB4 | Wht-Vio | BCD channel selector, D. Pulled up to +5V through a 10k resistor. |  |
|  | 5 | 15 |  | Mic-Lo | Shield for 16 | Microphone input ground/shield connection |  |
|  | 6 | 2 |  | Vol/Sq Lo | Shield for 1,4,5 | Volume Squelch ground/shield connection |  |
|  | 7 | 4 |  | Vol/Sql Hi | Grn-cntr | Volume Squelch output (low impedance non-demphasized, non-squelched audio) |  |
|  | 8 | 5 |  | Vol Arm | Blu-cntr | Audio amplifier input from external volume control |  |
|  | 9 | 16 |  | Mic-Hi | SF22-Brn | Microphone input |  |
|  | 10 | 12 |  | !CG DIS!/serial pgm | Wht-Grn | Ground this line to disable Channel Guard and mute received audio based on carrier squelch only. Also used as serial control line during programming. |  |
|  | 11 | 14 |  | PTT | Wht | Push to talk. Ground to Transmit |  |
|  | 12 | 10 |  | CAS | Vio | Channel Activity indicator. Follows Carrier Squelch action only regardless of radio programming. 0 Volts when squelched, 9 volts (via a 10k pullup) when un-squelched. |  |
|  | 13 |  | 1 | PA Key | Org-Blu | Grounding this pin enables the audio amplifier but does not force open the received audio gate. Used to enable the speaker audio amplifier to generate an alert tone when the CCT timer expires. |  |
|  | 14 | 7 |  | Control A- | Shield for 3 | Negative power supply output. Leave this unconnected. |  |
|  | 15 | 1 |  | !SQ DIS!/SQ ARM | Gra - cntr | With J605 shunted between pins 2 & 3 this is the squelch arm input. With J605 shunted between pins 1 & 2, when this pin is grounded, all squelch action is overriden and received audio is passed to the speaker. |  |
|  | 16 | 13 |  | Ign A+ | Red | Positive power supply connection. Essentially a logic input to the radio but needs A+ for radio to work. |  |
|  | 17 | 3 |  | Alert tone/RX PA | Yel-Grn | Input to the audio amplifier for time out timer alert tone. This pin also has squelched volume squelch high audio on it, but also be aware that it is also DC Biased to 4.5 Volts so AC couple to this if you want to use it for something. |  |
|  | 18 | 17 |  | SPKR-Hi | #16-Grn | One side of the speaker. |  |
|  | 19 | 11 |  | A+ | #16-Yel | Positive power supply connection. This provides all power to the radio except for the RF Power Amplifier |  |
|  | 20 | 18 |  | SPKR-Lo | #16-Brn | The other side of the speaker. |  |
|  | 21 | 19 |  | A- | #16-Blk | Negative power supply connection |  |
|  | 22 |  | 8 | Spare | Yel-Vio | N/C |  |
|  | 23 |  | 7 | Spare | Grn | N/C |  |
|  | 24 |  | 13 | Spare | Yel-Brn | N/C |  |
|  | 25 |  | 15 | Spare | Wht-Yel | N/C |  |
|  | 26 |  | 6 | CG-hi/HL1 | Wht-cntr | Option specific. DATA IN for some radios. |  |
|  | 27 | 6 |  | *SQ DIS/ HL2 | Org | Option specific. |  |
|  | 28 |  | 5 | CG-lo, HL3 | shield for 5 | Option specific. |  |
|  | 29 |  | 16 | !STORE! | Wht-Org | Used for radio programming. Grounding this line will cause the EEPROM to save data to non-volitale memory. |  |
|  | 30 |  | 19 | FB5/Mode A/*B | Wht-Gra | Used to select between two EEPROM banks. Also used as serial data line for radio programming. |  |
|  | 31 |  | 3 | Spare, HL6 | Yel-Vio | Option specific. |  |
|  | 32 |  | 14 | Adv Change, HL7 | Yel-Blk | Used by scan heads to advance channel. Also used as serial data clock line for radio programming. |  |
|  | 33 |  | 2 | Spare, HL8 | Org-Grn | Option specific. |  |
|  | 34 |  | 12 | !RX MUTE! | Yel-Blu | Ground to mute receiver audio... Side benefit is that it can be used as a Channel Guard decode indicate output. Open Collector output with a 10k pullup to +9 Volts. Grounded by Q721 when no Channel Guard tone is detected. |  |
|  | 35 |  | 4 | Spare | Org-Brn |  |  |
|  | 36 |  | 9 | !RESET! | Org | Used for radio programming. Grounding this pin will put the microprocessor into RESET. |  |
|  | 37 |  |  | Battery A+ | #8-Red | High current positive power supply connection. Provides power to RF Power Amplifier only. |  |
|  | 38 |  |  | Battery A- | #8-Blk | High current negative power supply connection. Provides power to RF Power Amplifier only. |  |
|  | S-550 Rear Panel Layout |  |  |  |  |  |  |
|  | 2 18 |  |  | MIC |  |  |  |
|  | P701 | P702 | P703 |  |  |  |  |
|  | 1 19 |  |  |  |  |  |  |
|  | Vehicle System Plug P701 |  |  |  |  |  |  |
|  |  |  | P703 | Function | To |  |  |
|  |  |  | 1 | A- | To Battery Negative |  |  |
|  |  |  | 2 | EXT SPKR-2 | To External Speaker |  |  |
|  |  |  | 3 |  |  |  |  |
|  |  |  | 4 | SPKR-2 | To Internal Speaker |  |  |
|  |  |  | 5 |  |  |  |  |
|  |  |  | 6 |  |  |  |  |
|  |  |  | 7 |  |  |  |  |
|  |  |  | 8 |  |  |  |  |
|  |  |  | 9 | A- | To Battery Negative |  |  |
|  |  |  | 10 | !CG Disable! | To Mic Hookswitch |  |  |
|  |  |  | 11 | A+ (Battery) | To Battery + via 5A fuse |  |  |
|  |  |  | 12 |  |  |  |  |
|  |  |  | 13 |  |  |  |  |
|  |  |  | 14 | A- | To Mic Hookswitch |  |  |
|  |  |  | 15 |  |  |  |  |
|  |  |  | 16 | Ign A+ (Battery) | To Battery + via 5A fuse |  |  |
|  |  |  | 17 | SPKR-1 | To Internal Speaker |  |  |
|  |  |  | 18 |  |  |  |  |
|  |  |  | 19 | EXT SPKR-1 | To External Speaker |  |  |
|  | NOTES |  |  |  |  |  |  |
|  | Connect +13.8V to pins 16 and 19 for all radios. If radio will be used as a transmitter also connect pin 37. |  |  |  |  |  | ……… |
|  | Connect power ground to pin 21. If radio will be used as a transmitter also connect pin 38. |  |  |  |  |  |  |
|  | For single channel use, leave pins 1 thru 4 floating and program channel 16 (memory location's 0xF0 - 0xFF). |  |  |  |  |  |  |
|  | - For Transmit applications - |  |  |  |  |  |  |
|  | Connect transmit audio to Pin 9. Transmit audio ground to pin 5. |  |  |  |  |  |  |
|  | Connect PTT to Pin 12 |  |  |  |  |  |  |
|  | - For Receive applications - |  |  |  |  |  |  |
|  | Unsquelched, low impedance, discriminator (flat) audio output is found on pin 7. Ground/Sheild to pin 6 if needed. |  |  |  |  |  |  |
|  | CAS (carrier squelch indicate) can be found on pin 12. |  |  |  |  |  |  |
|  | Channel Guard Decode indicate can be found on pin 34. |  |  |  |  |  |  |
|  | Use internal squelch control by removing shunt on J605, OR shunt J605 2&3 and install a 10k pot between |  |  |  |  |  |  |
|  |  | Pin 7 and 6 with the wiper connected to pin 15 to act as a squelch control. |  |  |  |  |  |
|  | For a combined Channel Guard Decode and Carrier Squelch logic output there are a couple choices. |  |  |  |  |  |  |
|  | If an internal squelch control is used, then shunt J605 1&2. Connect pin 15 (*SQ Disable) to the base of an NPN transistor through |  |  |  |  |  |  |
|  |  | a 10k resistor. Connect the transistor's emitter to ground, and use the collector of the transistor as the logic output. |  |  |  |  |  |
|  |  | Use a pullup resistor to the appropriate voltage if your controller does not provide this already. |  |  |  |  |  |
|  | If an external squelch control is used, then pin 15 is used as the Squelch Arm input to the radio and you will need to bring the |  |  |  |  |  |  |
|  |  | *SQ Disable signal out to another pin. The easiest way to do that would be to add a wire between P605 pin 1 and |  |  |  |  |  |
|  |  | one of the spare pins on the radio connector such as Pin 33 via the HL8 solder connection. You could also include |  |  |  |  |  |
|  |  | the buffer transistor inside the radio using one of the option pins to bring out the buffered signal as well. |  |  |  |  |  |